COSHH Assessment

AdBlue (Diesel Exhaust Fluid)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_01 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ Adblue.pdf (SDS) — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | AdBlue (Diesel Exhaust Fluid) |
| --- | --- |
| Supplier | Various automotive suppliers (industry-standard AUS 32 grade to ISO 22241) |
| Active ingredients / CAS | Urea solution 32.5% w/w — CAS 57-13-6 (urea) |
| Physical form | Clear liquid |
| Appearance / odour | Colourless to pale yellow, slight ammonia odour |
| Use | Diesel exhaust after-treatment (Selective Catalytic Reduction systems) |

2. Hazard Classification

Overall classification: Not classified as hazardous under CLP Signal word: Not required — product not classified. Hazard statements (H-codes): No hazard classification required under CLP. Precautionary statements (P-codes): No specific precautionary statements applicable. GHS pictograms: No GHS pictograms apply — product not classified as hazardous.

3. Persons at Risk

•  Vehicle maintenance staff

•  Plant operators

•  Refuelling personnel

4. Routes of Exposure

•  Inhalation — Low; not volatile at ambient temperature

•  Skin contact — Low; may cause mild irritation on prolonged contact

•  Eye contact — Slight irritation on direct contact

•  Ingestion — Low; seek medical advice if significant quantity swallowed

5. Workplace Exposure Limits

No UK Workplace Exposure Limits (HSE EH40/2005) assigned to this product or its constituents.

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Proprietary filling equipment

•  Store in bunded area

•  Spill kit within 5 m of dispensing point

Administrative controls

•  Induction training on filling procedures

•  Equipment inspection monthly

•  Clean up spills immediately

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

•  Coveralls if bulk handling

7. Storage

| Temperature | −11 °C to 30 °C |
| --- | --- |
| Location | Cool, dry area |
| Avoid | Direct sunlight, heat sources, dissimilar metals (e.g. copper, aluminium) |
| Container | Original container |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for several minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air (unlikely hazard). |
| Ingestion | Rinse mouth. Drink water. Seek medical advice if unwell. |

Spillage

Contain with absorbent material. Collect in appropriate container. Wash area with water. Dispose via authorised waste contractor.

Emergency contacts

•  Emergency services — 999

•  NHS non-emergency — 111

•  AMWS supervisor — Jason May

•  HSQE support — Sean Ashton (Onyx Operations)

9. Waste Disposal

Non-hazardous waste. Dispose via authorised waste contractor. Standard waste transfer notes apply.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Low | With controls — Low (residual) |
| --- | --- |

11. Training Requirements

•  Filling procedures (van-mounted dispensing)

•  Spill response

•  Storage requirements

12. Monitoring & Review

•  Monthly storage-area inspection

•  Annual COSHH review

•  Equipment maintenance checks

13. References

•  POL_HSQE_08 — COSHH Policy

•  POL_HSQE_19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 — Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/