COSHH Assessment

Carplan All Seasons Screenwash

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_03 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ carplan-all-seasons-screenwash-5l~GPID_1000199998_COSHH_00.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Carplan All Seasons Screenwash |
| --- | --- |
| Supplier | Tetrosyl Ltd (Carplan brand) |
| Active ingredients / CAS | Isopropanol (propan-2-ol) + surfactants + dye — CAS: 67-63-0 (isopropanol) |
| Physical form | Liquid |
| Appearance / odour | Coloured liquid (typically blue/yellow) |
| Use | Vehicle windscreen washer fluid — neat or diluted |

2. Hazard Classification

Overall classification: Flammable Liquid Category 3 (depending on concentration) Signal word: Warning Hazard statements (H-codes) H226 Flammable liquid and vapour. H319 Causes serious eye irritation. H336 May cause drowsiness or dizziness. Precautionary statements (P-codes) P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P233 Keep container tightly closed. P280 Wear protective gloves/protective clothing/eye protection/face protection. P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P403+235 Store in a well-ventilated place. Keep cool. GHS pictograms • GHS02 — Flammable • GHS07 — Harmful / irritant

3. Persons at Risk

•  Drivers

•  Vehicle maintenance staff

4. Routes of Exposure

•  Inhalation: Vapour may cause dizziness if concentrated

•  Skin contact: Defats skin on repeated contact

•  Eye contact: Causes irritation

•  Ingestion: Harmful — do not swallow

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Propan-2-ol | 999 mg/m³ (400 ppm) | 1250 mg/m³ (500 ppm) |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Keep container sealed when not in use

Administrative controls

•  No smoking during filling

•  Use only intended dispensers

Personal Protective Equipment (PPE)

•  Nitrile gloves when handling concentrate

•  Safety glasses

7. Storage

| Temperature | Cool store |
| --- | --- |
| Location | Fire-resistant store or lockable cabinet |
| Avoid | Heat, sparks, open flames |
| Container | Original sealed container |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. Remove contaminated clothing. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | Rinse mouth. Do not induce vomiting. Seek medical advice immediately. |

Spillage

Eliminate ignition sources. Absorb with sand/vermiculite. Dispose as hazardous waste.

Fire

Flammable. Use foam, CO₂ or dry powder. Do not use water jet.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Hazardous waste — dispose via authorised waste contractor.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low |
| --- | --- |

11. Training Requirements

•  Flammable liquids handling

•  Fire response

12. Monitoring & Review

•  Monthly storage inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/