COSHH Assessment

Comma Eurolite 10W-40 Semi-Synthetic Motor Oil

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_05 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ EUL-L.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Comma Eurolite 10W-40 Semi-Synthetic Motor Oil |
| --- | --- |
| Supplier | Comma Oil & Chemicals Ltd |
| Active ingredients / CAS | Highly-refined mineral base oil + performance additives — CAS: 64742-54-7 (base oil) |
| Physical form | Viscous liquid |
| Appearance / odour | Amber/brown liquid |
| Use | Engine lubricant for petrol and diesel vehicles |

2. Hazard Classification

Overall classification: Not classified as hazardous (fresh oil). Used oil may contain hazardous combustion products. Signal word: Not required — product not classified. Hazard statements: No hazard classification required under CLP. Precautionary statements (P-codes) P264 Wash thoroughly after handling. P280 Wear protective gloves/protective clothing/eye protection/face protection. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • No GHS pictograms apply — product not classified as hazardous.

3. Persons at Risk

•  Vehicle maintenance staff

•  Plant operators

4. Routes of Exposure

•  Inhalation: Low — mist possible during pouring

•  Skin contact: Repeated/prolonged contact may cause dermatitis

•  Eye contact: Mild irritation

•  Ingestion: Low

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Drip trays under dispensing points

•  Bunded storage

Administrative controls

•  Avoid skin contact — used oil is a recognised carcinogen (see HSE INDG35)

•  Change contaminated clothing promptly

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

•  Overalls

7. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Dry store, bunded if bulk |
| Avoid | Strong oxidisers |
| Container | Original container |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. Apply emollient. Seek medical advice if dermatitis develops. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek medical advice. |

Spillage

Absorb with sand or proprietary oil absorbent. Dispose as special/hazardous waste.

Fire

Combustible if heated. Use foam, dry powder or CO₂.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Used engine oil is hazardous (EWC 13 02). Contain and send via authorised waste oil collector.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Low | With controls — Minimal |
| --- | --- |

11. Training Requirements

•  Safe oil handling

•  Used-oil segregation

•  Skin-care awareness (used oil carcinogen)

12. Monitoring & Review

•  Monthly storage-area inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/