COSHH Assessment

GT85 PTFE Lubricant Spray

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_06 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ 202262-GT85-MSDS.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | GT85 PTFE Lubricant Spray |
| --- | --- |
| Supplier | GT85 Ltd |
| Active ingredients / CAS | Petroleum distillates + PTFE + propellant (LPG / butane) — CAS: 64742-49-0 (naphtha light hydrotreated) |
| Physical form | Aerosol |
| Appearance / odour | Clear spray, solvent odour |
| Use | Penetrating lubricant / water-displacing spray for tools, chains and fittings |

2. Hazard Classification

Overall classification: Aerosol Category 1 (Extremely flammable aerosol), Aspiration Cat 1 Signal word: Danger Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H304 May be fatal if swallowed and enters airways. H315 Causes skin irritation. H336 May cause drowsiness or dizziness. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P102 Keep out of reach of children. P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P261 Avoid breathing dust/fume/gas/mist/vapours/spray. P271 Use only outdoors or in a well-ventilated area. P410+412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS02 — Flammable • GHS07 — Harmful / irritant • GHS08 — Health hazard (serious) • GHS09 — Environmental hazard

3. Persons at Risk

•  Operatives using hand tools

•  Vehicle maintenance staff

4. Routes of Exposure

•  Inhalation: Vapour/aerosol mist may cause dizziness and headache

•  Skin contact: May cause dermatitis on repeated contact

•  Eye contact: Mechanical irritation from propellant

•  Ingestion: Aspiration hazard — may be fatal if inhaled into lungs

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Naphtha/hydrocarbons | 1200 mg/m³ | — |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Use in well-ventilated area — open air preferred

Administrative controls

•  No smoking during use

•  Do not pierce or burn can — even when empty

Personal Protective Equipment (PPE)

•  Nitrile gloves for prolonged use

•  Safety glasses

•  Respiratory protection if used in confined space

7. Storage

| Temperature | Below 50 °C |
| --- | --- |
| Location | Cool, ventilated store; protect from sunlight |
| Avoid | Heat, sparks, ignition sources, temperatures above 50 °C |
| Container | Original aerosol can |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. Seek medical attention if symptoms persist. |
| Ingestion | Do NOT induce vomiting (aspiration risk). Seek immediate medical attention. |

Spillage

Eliminate ignition sources. Ventilate. Absorb with sand/vermiculite. Dispose as hazardous waste.

Fire

Extremely flammable aerosol. Use foam, CO₂ or dry powder. Cool exposed cans with water — risk of explosion.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Empty aerosols are hazardous (EWC 16 05 04*). Dispose via authorised contractor.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low |
| --- | --- |

11. Training Requirements

•  Aerosol handling

•  Fire response

•  Confined-space awareness

12. Monitoring & Review

•  Weekly van-pack inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/