COSHH Assessment

Hy-Ram Chlorine Disinfection Tablets

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_07 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ 1084761.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Hy-Ram Chlorine Disinfection Tablets |
| --- | --- |
| Supplier | Hy-Ram Engineering Ltd |
| Active ingredients / CAS | Sodium dichloroisocyanurate (NaDCC) — CAS: 2893-78-9 |
| Physical form | Effervescent tablets |
| Appearance / odour | White tablets, chlorine odour when dissolved |
| Use | Disinfection of potable water mains following repair/installation |

2. Hazard Classification

Overall classification: Acute Toxicity (oral) Cat 4, Eye Irritation Cat 2, STOT SE 3 (respiratory), Aquatic Acute 1 Signal word: Warning Hazard statements (H-codes) H302 Harmful if swallowed. H319 Causes serious eye irritation. H335 May cause respiratory irritation. H410 Very toxic to aquatic life with long lasting effects. EUH031 Contact with acids liberates toxic gas. Precautionary statements (P-codes) P261 Avoid breathing dust/fume/gas/mist/vapours/spray. P264 Wash thoroughly after handling. P271 Use only outdoors or in a well-ventilated area. P273 Avoid release to the environment. P280 Wear protective gloves/protective clothing/eye protection/face protection. P301+312 IF SWALLOWED: Call a POISON CENTRE or doctor/physician if you feel unwell. P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS07 — Harmful / irritant • GHS09 — Environmental hazard

3. Persons at Risk

•  Water-main operatives

•  Mains-commissioning personnel

4. Routes of Exposure

•  Inhalation: Dust/fume may irritate respiratory tract

•  Skin contact: Irritation on prolonged contact

•  Eye contact: Causes irritation

•  Ingestion: Harmful if swallowed

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Chlorine (released) | 0.5 ppm | 1 ppm |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Dissolve only in approved chlorination rig

•  Never add water to tablet — add tablet to water

Administrative controls

•  Use only for purpose specified

•  Keep away from acids

•  One tablet per intended volume only

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Chemical goggles

•  Dust mask when handling dry tablets

7. Storage

| Temperature | Ambient, dry |
| --- | --- |
| Location | Original container only. Locked cabinet. |
| Avoid | Moisture, acids, ammonia, combustibles, direct sunlight |
| Container | Original sealed drum — do not decant |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | Rinse mouth. Drink water. Do NOT induce vomiting. Seek medical attention. |

Spillage

Dry spill: sweep carefully (avoid dust) into container for disposal. Wet spill: dilute with large volumes of water. Do not return to container.

Fire

Not combustible but supports combustion. Use water spray. DO NOT use dry powder containing ammonium compounds.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Hazardous waste. Dispose via authorised chemical-waste contractor.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — High | With controls — Low |
| --- | --- |

11. Training Requirements

•  Water-main chlorination procedure

•  COSHH briefing

•  Spill response

12. Monitoring & Review

•  Weekly visual stock check

•  Expiry-date check monthly

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/