COSHH Assessment

Lapwing Line-Marking Paint (White)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_08 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ LMY-SDS.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Lapwing Line-Marking Paint (White) |
| --- | --- |
| Supplier | Lapwing UK Ltd |
| Active ingredients / CAS | Acrylic polymer, titanium dioxide (TiO₂) — CAS: 13463-67-7 (TiO₂) |
| Physical form | Aerosol paint |
| Appearance / odour | White liquid/aerosol |
| Use | Line-marking of excavations, site set-out and car parks |

2. Hazard Classification

Overall classification: Aerosol Cat 1 (flammable aerosol) — water-based variants may differ Signal word: Danger Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H319 Causes serious eye irritation. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P280 Wear protective gloves/protective clothing/eye protection/face protection. P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P410+412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS02 — Flammable • GHS07 — Harmful / irritant • GHS09 — Environmental hazard

3. Persons at Risk

•  Line-marking operatives

•  Highway maintenance staff

•  Site personnel

4. Routes of Exposure

•  Inhalation: Mist may cause respiratory irritation

•  Skin contact: Minor staining; mild irritation

•  Eye contact: Mist causes irritation

•  Ingestion: Unlikely in normal use

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Titanium dioxide (inhalable) | 10 mg/m³ | — |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Use outdoors or in well-ventilated area

•  Traffic management in place

Administrative controls

•  Check weather before use

•  No smoking / hot work nearby

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

•  High-visibility clothing

•  Coveralls/disposable apron

7. Storage

| Temperature | Below 50 °C |
| --- | --- |
| Location | Cool, ventilated store |
| Avoid | Heat, sparks, frost, direct sunlight |
| Container | Original aerosol can |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice if irritation persists. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek medical advice. |

Spillage

Eliminate ignition sources. Absorb with inert material. Dispose as hazardous waste.

Fire

Flammable aerosol. Use foam, CO₂ or dry powder. Cool exposed cans with water.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Empty aerosols are hazardous (EWC 16 05 04*).

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low |
| --- | --- |

11. Training Requirements

•  Aerosol handling

•  Line-marking safe working

12. Monitoring & Review

•  Weekly van-pack inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/