COSHH Assessment

Traffic Film Remover (Orbit — Würth)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_10 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ C312.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Traffic Film Remover (Orbit — Würth) |
| --- | --- |
| Supplier | Würth UK Ltd |
| Active ingredients / CAS | Alkaline cleaner — contains sodium hydroxide + anionic surfactants — CAS: 1310-73-2 (NaOH) |
| Physical form | Liquid concentrate |
| Appearance / odour | Clear/pale-coloured liquid |
| Use | Exterior vehicle pre-wash for removal of road film |

2. Hazard Classification

Overall classification: Skin Corrosive Cat 1B, Eye Damage Cat 1 Signal word: Danger Hazard statements (H-codes) H314 Causes severe skin burns and eye damage. Precautionary statements (P-codes) P260 Do not breathe dust/fume/gas/mist/vapours/spray. P264 Wash thoroughly after handling. P280 Wear protective gloves/protective clothing/eye protection/face protection. P301+330+331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. P303+361+353 IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water. P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P310 Immediately call a POISON CENTRE or doctor/physician. P405 Store locked up. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS05 — Corrosive

3. Persons at Risk

•  Vehicle-cleaning operatives

4. Routes of Exposure

•  Inhalation: Mist irritates respiratory tract

•  Skin contact: Severe burns

•  Eye contact: Severe damage — irreversible possible

•  Ingestion: Severe corrosive effects

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Sodium hydroxide (mist) | 2 mg/m³ (15-min STEL) | — |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Use in designated wash bay with bund

•  Eyewash station within 10 m

•  Pressure washer with fan nozzle to reduce splash-back

Administrative controls

•  Always dilute per instructions — never use neat

•  Never mix with acids or other chemicals

Personal Protective Equipment (PPE)

•  Nitrile or PVC chemical gloves

•  Chemical goggles or face shield

•  Chemical-resistant apron

•  Wellingtons

7. Storage

| Temperature | 5–25 °C |
| --- | --- |
| Location | Bunded area, away from acids |
| Avoid | Acids, aluminium, zinc, organics |
| Container | Original sealed container |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for at least 15 minutes. Remove contact lenses. Seek immediate medical attention. |
| Skin | Remove contaminated clothing. Wash with plenty of water for 15 minutes. Seek medical attention. |
| Inhalation | Move to fresh air. Seek medical advice if symptoms persist. |
| Ingestion | DO NOT induce vomiting. Rinse mouth. Give water if conscious. Seek immediate medical attention. |

Spillage

Dilute with water. Neutralise with dilute acid (e.g. citric acid). Absorb with inert material. Dispose as hazardous waste.

Fire

Not flammable.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Hazardous waste (alkaline). Dispose via authorised contractor.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — High | With controls — Low |
| --- | --- |

11. Training Requirements

•  Corrosive chemical handling

•  Spill response

•  PPE fit

12. Monitoring & Review

•  Weekly PPE condition check

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/