COSHH Assessment

Blackfriar Temporary Line-Marking Paint — Yellow

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_11 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ AE0220002-0003-0004-0005-0009-0359-Line-Marking-Paint-Blackfriar-gb.pdf — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | Blackfriar Temporary Line-Marking Paint — Yellow |
| --- | --- |
| Supplier | Blackfriar Paints Ltd |
| Active ingredients / CAS | Hydrocarbon solvents + pigment + propellant — CAS: Mixture — see SDS |
| Physical form | Aerosol |
| Appearance / odour | Yellow spray |
| Use | Temporary yellow line-marking for site demarcation and hazards |

2. Hazard Classification

Overall classification: Aerosol Cat 1, Aspiration Cat 1, STOT SE 3 Signal word: Danger Hazard statements (H-codes) H222 Extremely flammable aerosol. H229 Pressurised container: may burst if heated. H304 May be fatal if swallowed and enters airways. H315 Causes skin irritation. H336 May cause drowsiness or dizziness. H411 Toxic to aquatic life with long lasting effects. Precautionary statements (P-codes) P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P211 Do not spray on an open flame or other ignition source. P251 Do not pierce or burn, even after use. P261 Avoid breathing dust/fume/gas/mist/vapours/spray. P280 Wear protective gloves/protective clothing/eye protection/face protection. P410+412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS02 — Flammable • GHS07 — Harmful / irritant • GHS08 — Health hazard (serious) • GHS09 — Environmental hazard

3. Persons at Risk

•  Line-marking operatives

•  Site personnel

4. Routes of Exposure

•  Inhalation: Vapour/mist — may cause dizziness

•  Skin contact: Irritation on prolonged contact

•  Eye contact: Irritation

•  Ingestion: Aspiration hazard

5. Workplace Exposure Limits

_No UK Workplace Exposure Limits (HSE EH40/2005) assigned to this product or its constituents._

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Use in open air wherever possible

Administrative controls

•  No smoking or hot work nearby

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

7. Storage

| Temperature | Below 50 °C |
| --- | --- |
| Location | Cool ventilated store |
| Avoid | Heat, sparks, direct sunlight |
| Container | Original aerosol can |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for 15 minutes. Seek medical advice. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. |
| Ingestion | Do not induce vomiting. Seek immediate medical attention. |

Spillage

Eliminate ignition sources. Absorb with inert material. Dispose as hazardous waste.

Fire

Flammable aerosol. Use foam, CO₂ or dry powder.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Empty aerosols are hazardous (EWC 16 05 04*).

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low |
| --- | --- |

11. Training Requirements

•  Aerosol handling

12. Monitoring & Review

•  Weekly van-pack inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/