COSHH Assessment

STIHL HP 2-Stroke Engine Oil (100 ml)

| Document Information |  |
| --- | --- |
| Document Reference | COSHH_22 |
| Issue Number | 2 |
| Issue Date | 1 June 2026 |
| Next Review | 1 June 2027 |
| Assessor | Sean Ashton (Onyx Operations) |
| Approved By | Aaron Mason, Director |

Key references ▸ SDS not available — Manufacturer Safety Data Sheet ▸ Van Packs portal — COSHH section — Synchronised copy carried by every operative

1. Product Identification

| Product name | STIHL HP 2-Stroke Engine Oil (100 ml) |
| --- | --- |
| Supplier | Andreas Stihl Ltd / CID Group (supplied unit) |
| Active ingredients / CAS | Petroleum mineral oil lubricant base + proprietary performance additives — CAS: 64742-65-0 (solvent-refined heavy paraffinic oil) + others |
| Physical form | Liquid |
| Appearance / odour | Red-tinted oil, mild hydrocarbon odour |
| Use | 2-stroke fuel mix additive (50:1 ratio) for petrol-powered Stihl tools (disc-cutters, saws, breakers) |

2. Hazard Classification

Overall classification: Aspiration Toxicity Cat 1, Aquatic Chronic 3 Signal word: Danger Hazard statements (H-codes) H304 May be fatal if swallowed and enters airways. H412 Harmful to aquatic life with long lasting effects. Precautionary statements (P-codes) P102 Keep out of reach of children. P264 Wash thoroughly after handling. P273 Avoid release to the environment. P301+310 IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. P331 Do NOT induce vomiting. P405 Store locked up. P501 Dispose of contents/container to an authorised waste contractor in accordance with local regulations. GHS pictograms • GHS08 — Health hazard (serious)

3. Persons at Risk

•  Operatives using Stihl 2-stroke tools

•  Fuel-mixing personnel

4. Routes of Exposure

•  Inhalation: Low — not volatile at ambient temperature

•  Skin contact: Repeated/prolonged contact may cause dermatitis

•  Eye contact: Mild irritation on direct contact

•  Ingestion: May be fatal if swallowed and enters airways (aspiration hazard)

5. Workplace Exposure Limits

| Substance | 8-hr TWA | 15-min STEL |
| --- | --- | --- |
| Oil mist, mineral | 5 mg/m³ | 10 mg/m³ |

6. Control Measures — Hierarchy of Controls

Elimination / substitution

Evaluated where practicable — see Section 12.

Engineering controls

•  Mix fuel outdoors where possible

•  Use proprietary fuel-mixing bottle (pre-marked 50:1 graduations)

•  Drip tray for decanting

Administrative controls

•  Mix only sufficient fuel for the day's work (do not store mixed fuel long-term)

•  Label all mixed fuel containers

•  Keep away from food/drink

Personal Protective Equipment (PPE)

•  Nitrile gloves

•  Safety glasses

7. Storage

| Temperature | Ambient |
| --- | --- |
| Location | Dry store, away from food and drink |
| Avoid | Direct heat, strong oxidisers, food/drink |
| Container | Original 100 ml bottle — keep tightly closed |

8. Emergency Procedures

First aid

| Route | Action |
| --- | --- |
| Eyes | Rinse with water for several minutes. |
| Skin | Wash with soap and water. |
| Inhalation | Move to fresh air. |
| Ingestion | **DO NOT induce vomiting** (aspiration risk). Rinse mouth. Seek immediate medical attention — POISON CENTRE: 111 (NHS) or 999. |

Spillage

Absorb with sand, vermiculite or proprietary oil absorbent. Dispose as hazardous waste. Do not release to drains or watercourses.

Fire

Combustible when heated. Use foam, dry powder or CO₂.

Emergency contacts

•  Emergency services: 999

•  NHS non-emergency: 111

•  AMWS supervisor: Jason May

•  HSQE support: Sean Ashton (Onyx Operations)

9. Waste Disposal

Hazardous waste (EWC 13 02 05*). Dispose via authorised waste-oil contractor.

10. Risk Rating

Risk = Severity × Likelihood (S × L = R). See POL_HSQE_21 Risk Assessment policy for the full scoring matrix.

| Before controls — Moderate | With controls — Low |
| --- | --- |

11. Training Requirements

•  Fuel-mixing procedure

•  Spill response

•  Hand-arm vibration awareness (tool operation)

12. Monitoring & Review

•  Weekly van-pack inspection

•  Annual COSHH review

13. References

•  POLHSQE08 — COSHH Policy

•  POLHSQE19 — Personal Protective Equipment (PPE) Policy

•  HSE COSHH Regulations 2002 (SI 2002/2677, as amended)

•  HSE EH40/2005 Workplace Exposure Limits

•  CLP Regulation (EC) No 1272/2008 (as retained in UK law)

This assessment must be reviewed annually or sooner if circumstances change. For the full management-system context (SOPs, Appendices, Audit records) see https://amwaterservices.onyxoperations.co.uk/